AUTOMOTIVE TEST ENGINEERING PLATFORM

Volume V

Diagnostics Engineering Workbook

Architecture, implementation record, verification strategy, and engineering evidence

| Document version | 0.6.0 |
| --- | --- |
| Baseline date | 3 September 2026 |
| Status | Implemented increments: V-1 through V-6 |
| Language | English |

A living technical record for architecture, software engineering, testing, operations, and portfolio evidence.

# Contents

This contents list follows the workbook's maintained section structure. Word's Navigation Pane can also be used because all sections use semantic heading styles.

- Document Purpose

- Document Control

- Revision History

- Scope and Boundaries

- Architecture

- Domain Model

- Public API and Security

- UDS Semantics

- Error and Consistency Contract

- Engineering Decisions

- Verification Catalogue

- Implemented Evidence

- Risks and Technical Debt

- Roadmap

- Study Exercises

- V-6 Review Worksheet

# 1. Document Purpose

This workbook records the architecture, requirements, engineering decisions, implementation evidence, verification strategy, risks, and study exercises for the Diagnostics volume.

# 2. Document Control

| Field | Value |
| --- | --- |
| Volume | V - Diagnostics |
| Baseline | Increments V-1 through V-6 |
| Architecture style | ECU-scoped diagnostic aggregate and transactional domain service |
| Primary runtime | Python 3.12, FastAPI, SQLAlchemy, PostgreSQL |
| Quality gates | pytest, Ruff, strict mypy, Alembic, integration CI, DOCX render and accessibility audit |

# 3. Revision History

| Release | Date | Change |
| --- | --- | --- |
| 0.1.0 | 2026-08-25 | Added versioned UDS sessions, DTC storage, services 0x10, 0x19, and 0x14, stable NRC evidence, RBAC, audit, outbox, migration, APIs, and deterministic tests. |
| 0.2.0 | 2026-08-25 | Added a typed DID catalogue, UDS 0x22/0x2E, session authorization, value/version constraints, minimized evidence, migration, APIs, and tests. |
| 0.3.0 | 2026-08-25 | Added a bounded routine catalogue, UDS Routine Control 0x31, deterministic logical-time execution, start/stop/result subfunctions, versioned replay, minimized evidence, migration, APIs, and tests. |
| 0.4.0 | 2026-08-28 | Added UDS Security Access 0x27, deterministic level-1 seed/key exchange, logical-time expiry and lockout, idempotent negative evidence, secret minimization, migration, APIs, and tests. |
| 0.5.0 | 2026-08-29 | Added UDS ECU Reset 0x11, cross-volume ECU lifecycle orchestration, reset/session/security version checks, deterministic logical time, exact replay, RBAC, atomic audit/outbox evidence, API, and tests. |
| 0.6.0 | 2026-09-03 | Added bounded UDS firmware download 0x34/0x36/0x37, ordered block transfer, SHA-256 verification, firmware activation, payload minimization, migration, APIs, and tests. |

# 4. Scope and Boundaries

## 4.1 Included

- One current diagnostic session per ECU.

- Default, programming, and extended session types.

- Explicit UDS request and positive-response service identities.

- Stable negative response codes within the global ATEP error contract.

- Six-hex-digit DTC storage, status masks, severity, occurrence tracking, and logical timestamps.

- Bounded snapshot evidence and safe DTC pagination/filtering.

- Exact command replay and stable changed-reuse conflict behavior.

- Independent diagnostic read/manage permissions.

- Atomic state, evidence, audit, and transactional outbox writes.

- ECU-scoped typed DID catalogue with boolean, integer, decimal, and string values.

- Session-authorized UDS Read/Write Data by Identifier (0x22 and 0x2E).

- DID type, range, length, catalogue-size, request-size, and optimistic-version bounds.

- ECU-scoped routine definitions and independent versioned execution state.

- UDS Routine Control (0x31) start, stop, and request-results subfunctions.

- Active-session authorization and deterministic completion from ECU logical time.

- Bounded parameters, results, duration, catalogue size, and optimistic versions.

- Level-1 requestSeed/sendKey Security Access in programming and extended sessions.

- Three-attempt lockout, seed expiry, and required delay based on ECU logical time.

- Masked raw-key input, digest-only protected state, and seed/key-minimized shared evidence.

- Exact replay for accepted and denied Security Access commands.

- UDS ECU Reset hard, key-off/on, and soft subfunctions through the existing ECU lifecycle.

- Atomic restoration of default session, security level zero, and empty challenge/lockout state.

- Reset/session/security optimistic versions, session/security policy, and exact reset replay.

- ECU-scoped firmware-transfer state and UDS Request Download, Transfer Data, and Request Transfer Exit.

- Programming-session and level-1 security policy, 64-KiB image and 256-byte block bounds.

- Ordered block counters, SHA-256 image verification, atomic firmware activation, and exact replay.

## 4.2 Deferred

- ISO-TP segmentation, CAN transport binding, DoIP, and physical adapters.

- OBD-II mode/PID compatibility and fleet-scale remote diagnostics.

# 5. Architecture

The atep.diagnostics module sits above the ECU aggregate and below future CAN/DoIP transport adapters. It stores diagnostic protocol state without taking ownership of ECU memory, signals, or fault generation. This preserves a clean distinction between a simulated internal fault and a DTC that an external tester can query.

Request flow: authenticated client -> FastAPI router -> diagnostic RBAC -> vehicle/ECU lookup -> diagnostic service -> PostgreSQL state and evidence + audit + transactional outbox -> typed response.

# 6. Domain Model

DiagnosticSessionState stores ECU ownership, current session, security level, optimistic version, and ECU logical time. It started at zero in V-1; V-4 can raise it to level 1 after a successful Security Access exchange, while every session change resets it to zero.

DiagnosticCommand stores ECU-scoped command identity, UDS service ID, canonical request, result, session versions, actor, and timestamps. It supports exact replay for mutating services.

DiagnosticTroubleCode stores ECU ownership, six-digit hexadecimal code, ISO-style status mask, severity, description, occurrence count, first/last ECU logical time, bounded snapshot, and version.

DiagnosticDataIdentifier stores an ECU-local 16-bit identifier, engineering metadata, declared type, unit, readable/writable sessions, constrained scalar value, and optimistic version. The catalogue is capped at 128 entries per ECU; this is a semantic data surface, not raw memory access.

DiagnosticRoutine stores an ECU-local 16-bit identifier, engineering metadata, allowed sessions, bounded execution duration, stop capability, bounded scalar result template, and definition version. DiagnosticRoutineState independently stores idle/running/completed/stopped status, invocation count, logical timestamps, protected input/result values, and an optimistic version. The catalogue is capped at 64 routines per ECU.

DiagnosticSecurityState stores one ECU-local challenge counter, expected-key digest, seed expiry, invalid-attempt count, logical lockout deadline, target level, and optimistic version. It never stores a raw key. The current unlocked security level remains on DiagnosticSessionState, so a session change can reset authorization without rewriting challenge history.

DiagnosticFlashState stores one ECU-local transfer lifecycle: address/size negotiation, target firmware and ECU version, maximum block length, next sequence counter, received byte count, protected transient image bytes, final SHA-256 digest, status, and optimistic version. Completed transfers purge raw bytes after verification.

# 7. Public API and Security

| Method | Path | Permission | Purpose |
| --- | --- | --- | --- |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/session | diagnostics:read | Retrieve or initialize the default session. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/session-control | diagnostics:manage | Execute UDS service 0x10. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dtcs | diagnostics:manage | Report or update a simulated DTC. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dtcs | diagnostics:read | Execute the 0x19 query boundary with safe pagination. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dtcs/{code} | diagnostics:read | Retrieve one DTC. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dtcs/clear | diagnostics:manage | Execute 0x14 for group FFFFFF. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dids | diagnostics:manage | Define one typed DID. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dids | diagnostics:read | List the bounded ECU DID catalogue. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dids/{identifier} | diagnostics:read | Retrieve one DID definition and value. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dids/read | diagnostics:read | Execute UDS 0x22 for up to 16 DIDs. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/dids/{identifier}/write | diagnostics:manage | Execute session-authorized UDS 0x2E. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/routines | diagnostics:manage | Define one bounded routine. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/routines | diagnostics:read | List the bounded routine catalogue. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/routines/{identifier} | diagnostics:read | Retrieve one routine and its state. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/routines/{identifier}/control | diagnostics:manage | Execute UDS 0x31 start, stop, or result request. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/security-access/state | diagnostics:read | Retrieve non-secret version, attempt, lockout, and challenge status. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/security-access | diagnostics:manage | Execute UDS 0x27 requestSeed or sendKey. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/ecu-reset | diagnostics:manage | Execute UDS 0x11 through the ECU lifecycle. |
| GET | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/flash/state | diagnostics:read | Retrieve bounded transfer progress and digest metadata. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/flash/request-download | diagnostics:manage | Negotiate UDS 0x34 firmware transfer. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/flash/transfer-data | diagnostics:manage | Submit one ordered UDS 0x36 block. |
| POST | /api/v1/vehicles/{vehicle_id}/ecus/{ecu_id}/diagnostics/flash/request-transfer-exit | diagnostics:manage | Verify and activate through UDS 0x37. |

# 8. UDS Semantics

## 8.1 Diagnostic Session Control (`0x10`)

The service locks or initializes the ECU session state, verifies expected version, applies the new session, resets security level, copies ECU logical time, increments version, and persists command evidence. The positive response service ID is 0x50.

## 8.2 Read DTC Information (`0x19`)

The HTTP query is a transport-neutral representation of the read-DTC boundary. It orders by code, limits each page to 200 records, supports a non-zero status-mask intersection filter, and emits typed DTC records. A future wire adapter will encode the appropriate UDS subfunction and PDU.

## 8.3 Clear Diagnostic Information (`0x14`)

V-1 supports only the all-DTC group FFFFFF. The service removes matching ECU DTC records within the same transaction as versioned command, audit, and outbox evidence. The positive response service ID is 0x54; unsupported groups return NRC 0x31.

## 8.4 Read Data by Identifier (`0x22`)

One command reads one to sixteen unique 16-bit identifiers. Every DID must exist and authorize the active diagnostic session. The result preserves typed values for the client and exact replay; the positive response identity is 0x62. Audit and outbox records contain identifiers and versions, not values.

## 8.5 Write Data by Identifier (`0x2E`)

A write requires diagnostics:manage, a writable DID, an allowed active session, matching session and DID versions, and a value satisfying the declared type and constraints. It increments only the DID version and returns 0x6E identity. Exact retry returns the persisted result without a second mutation.

## 8.6 Routine Control (`0x31`)

Subfunctions 0x01, 0x02, and 0x03 represent startRoutine, stopRoutine, and requestRoutineResults. Every request requires an allowed active session plus matching session and routine-state versions. Start records the ECU logical start/completion timestamps and bounded parameters. Stop is accepted only for a running routine that declares stop support. A result request deterministically promotes a running routine to completed when ECU logical time reaches the target and returns the protected result template. Exact retries replay persisted command evidence without another transition; the positive response identity is 0x71.

## 8.7 Security Access (`0x27`)

V-4 supports level-1 requestSeed (0x01) and sendKey (0x02) in programming and extended sessions. A seed is deterministically generated from ECU identity, challenge counter, logical time, and state version; it expires after 30,000 logical milliseconds. The raw key is accepted as a masked 16-character secret and immediately reduced to SHA-256 for constant-time comparison. Three invalid keys produce NRC 0x36 and a 10,000 logical-millisecond delay; attempts during that delay return NRC 0x37. sendKey without a current challenge returns NRC 0x24, and an ordinary invalid key returns NRC 0x35. Successful access raises the session security level to 1 and uses positive response identity 0x67. The key derivation helper is intentionally a transparent, deterministic simulator fixture and must not be treated as production ECU cryptography.

## 8.8 ECU Reset (`0x11`)

V-5 supports hardReset (0x01), keyOffOnReset (0x02), and softReset (0x03). Every request requires programming or extended session; hard and key-off/on reset additionally require security level 1. The command checks ECU, session, and Security Access versions before invoking the existing Volume III ECU reset lifecycle. The lifecycle owns operational state, memory policy, fault preservation, boot count, ECU version, reset duration, and logical-time advancement. Within the same transaction, the diagnostic layer restores default session and security level zero, invalidates seed/attempt/lockout state, increments diagnostic versions, and records UDS evidence. The positive response identity is 0x51. Exact retry replays stored results without a second reset.

## 8.9 ECU Flash (`0x34`, `0x36`, `0x37`)

V-6 models flashing as a semantic, transport-neutral pipeline. Request Download requires the programming session, security level 1, matching ECU/session/security versions, an address range inside the 16-bit ECU space, and an image no larger than 65,536 bytes. Its positive response is 0x74 and negotiates a 256-byte maximum block length. Transfer Data (0x76) accepts only the expected byte counter, wraps 255 to 0, prevents overflow, and advances an independent transfer version. Command and shared evidence retain block size and SHA-256, never raw firmware bytes. Request Transfer Exit (0x77) requires the exact byte count, matching transfer/ECU versions, and constant-time SHA-256 verification before changing the ECU profile version and ECU optimistic version. Completion purges the assembled image bytes while retaining digest and bounded metadata. Every accepted mutation, command record, audit record, and outbox event shares one transaction.

# 9. Error and Consistency Contract

| Condition | Stable code | HTTP | UDS evidence |
| --- | --- | --- | --- |
| Changed reuse of a diagnostic command ID | diagnostic_command_conflict | 409 | Command identity conflict before PDU execution |
| Stale session version | diagnostic_contract_invalid | 422 | NRC 0x22 conditions not correct |
| Unsupported DTC group | diagnostic_contract_invalid | 422 | NRC 0x31 request out of range |
| DID type/range violation | diagnostic_contract_invalid | 422 | NRC 0x31 request out of range |
| Stale DID or session version | diagnostic_contract_invalid | 422 | NRC 0x22 conditions not correct |
| DID forbidden in active session | diagnostic_contract_invalid | 422 | NRC 0x7F service not supported in active session |
| Routine forbidden in active session | diagnostic_contract_invalid | 422 | NRC 0x7F service not supported in active session |
| Stale routine or session version | diagnostic_contract_invalid | 422 | NRC 0x22 conditions not correct |
| Unsupported stop or unknown routine identifier | diagnostic_contract_invalid | 422 | NRC 0x31 request out of range |
| sendKey without a valid seed | diagnostic_contract_invalid | 422 | NRC 0x24 request sequence error |
| Invalid Security Access key | diagnostic_contract_invalid | 422 | NRC 0x35 invalid key |
| Third invalid key | diagnostic_contract_invalid | 422 | NRC 0x36 exceed number of attempts |
| Access during logical lockout | diagnostic_contract_invalid | 422 | NRC 0x37 required time delay not expired |
| Reset in default session | diagnostic_contract_invalid | 422 | NRC 0x7F service not supported in active session |
| Hard or key-off/on reset without level 1 | diagnostic_contract_invalid | 422 | NRC 0x33 security access denied |
| Stale ECU, session, or security reset version | diagnostic_contract_invalid | 422 | NRC 0x22 conditions not correct |
| Flash outside programming session | diagnostic_contract_invalid | 422 | NRC 0x7F service not supported in active session |
| Flash without security level 1 | diagnostic_contract_invalid | 422 | NRC 0x33 security access denied |
| Transfer before download or exit before complete image | diagnostic_contract_invalid | 422 | NRC 0x24 request sequence error |
| Wrong block sequence counter | diagnostic_contract_invalid | 422 | NRC 0x73 wrong block sequence counter |
| Oversized block or image overflow | diagnostic_contract_invalid | 422 | NRC 0x13 invalid format/length |
| Stale transfer/ECU version or digest mismatch | diagnostic_contract_invalid | 422 | NRC 0x22 conditions not correct |
| Unknown ECU or DTC | Platform resource-not-found code | 404 | Adapter maps transport response later |
| Invalid body or pagination | request_validation_error | 422 | Rejected at the public contract boundary |

# 10. Engineering Decisions

## ADR-DG-001 - Separate ECU Faults from Diagnostic Trouble Codes

Internal ECU faults drive simulated behavior. DTCs are externally observable diagnostic evidence. The platform may bridge them, but neither representation silently overwrites the other.

## ADR-DG-002 - Preserve UDS Identity Without Premature Wire Coupling

Services, positive response IDs, and NRCs are explicit, while HTTP carries typed domain commands. ISO-TP and DoIP adapters can be added without rewriting persistence or business rules.

## ADR-DG-003 - Use ECU Logical Time

DTC first/last-seen time and session transitions use simulation time. Tests remain deterministic and do not sleep or depend on host clock speed.

## ADR-DG-004 - Minimize Observable Evidence

Snapshots may contain engineering measurements. They remain in protected DTC storage and are not copied into audit or outbox messages; those contain identifiers, status, counters, and versions.

## ADR-DG-005 - Model DIDs as Typed Semantic Data

DIDs describe automotive information explicitly instead of exposing arbitrary ECU memory. This supports stable validation, session policy, OpenAPI contracts, and future VHAL/CAN mappings.

## ADR-DG-006 - Preserve Values for Replay but Minimize Shared Evidence

Command results retain DID values so an exact retry is truthful. Audit and outbox records exclude those values and share only identifiers, counts, service identities, and versions.

## ADR-DG-007 - Separate Routine Definition from Execution State

Routine engineering metadata changes independently from each execution lifecycle. A one-to-one state record makes concurrency and lifecycle transitions explicit without rewriting definitions.

## ADR-DG-008 - Complete Routines from ECU Logical Time

Routine completion is evaluated against simulation time when the state is observed. No sleeping, host timer, worker loop, GPU, or cloud service is required, which keeps simulation and tests deterministic and inexpensive.

## ADR-DG-009 - Protect Routine Values in Shared Evidence

Parameters and results remain in protected state and command records for faithful replay. Audit and outbox payloads contain only identifiers, subfunctions, status, counters, timestamps, and versions to avoid distributing engineering measurements or sensitive routine inputs.

## ADR-DG-010 - Separate Security Challenge State from Session Authorization

Challenge lifecycle and invalid-attempt policy need their own optimistic version, while the unlocked level belongs to the current diagnostic session. This separation makes concurrency, session reset, and lockout behavior explicit.

## ADR-DG-011 - Persist Negative Attempts Before Returning an Error

Invalid-key attempts are security state changes, not validation-only failures. The service writes versioned command, audit, and outbox evidence atomically, and the endpoint commits that evidence before returning the stable UDS-aware error. Exact retries replay the same denial without another attempt increment.

## ADR-DG-012 - Use Transparent Simulator Cryptography with Strict Secret Hygiene

A deterministic public derivation supports reproducible study and black-box tests without paid services or secret provisioning. It is explicitly non-production. Raw keys are masked and never persisted; seeds and digests stay in protected state/command storage and never enter shared audit or outbox evidence.

## ADR-DG-013 - Orchestrate One ECU Reset Lifecycle Across Volumes

UDS 0x11 calls the reset behavior already owned by Volume III rather than reimplementing memory, boot, fault, timing, or operational-state rules. One database transaction contains both ECU-native and diagnostic evidence, followed by the session/security reset. This keeps the simulated ECU and external tester view consistent and makes rollback reliable.

## ADR-DG-014 - Apply Reset Authorization Before Lifecycle Mutation

Default-session reset is rejected. Hard and key-off/on reset additionally require level-1 Security Access; soft reset requires only programming or extended session. ECU, diagnostic-session, and security-state versions are validated so a stale tester cannot reset newer aggregate state.

## ADR-DG-015 - Keep Firmware Payloads Out of Shared Evidence

Raw firmware is required only while assembling an active transfer. Diagnostic command requests, audit, outbox, and logs contain block length and SHA-256 rather than payload bytes. Successful completion purges the assembled image and retains only bounded metadata and its digest.

## ADR-DG-016 - Activate Firmware Only at a Verified Transaction Boundary

Transfer blocks update only the flash aggregate. The ECU profile/version changes only when Request Transfer Exit verifies the declared byte count, transfer/ECU versions, and full-image SHA-256. Firmware activation, transfer completion, command evidence, audit, and outbox then commit together.

# 11. Verification Catalogue

| Test | Objective | Level |
| --- | --- | --- |
| DTC code shape | Reject anything other than six uppercase hexadecimal digits. | Schema |
| Status-mask bounds | Accept only values from zero through 255. | Schema |
| Snapshot bound | Reject more than 32 scalar snapshot entries. | Schema |
| Session types | Accept only default, programming, and extended. | Schema/OpenAPI |
| Initial session | Create one default session for an ECU without diagnostic state. | Service/integration |
| Session version | Reject a stale expected version with NRC 0x22. | Service/API |
| Session response | Expose request 0x10 and positive response 0x50. | Service/API |
| Exact replay | Return persisted command evidence without a second state change. | Service/integration |
| Changed reuse | Return stable diagnostic_command_conflict. | Service/API |
| DTC creation | Persist code, status, severity, description, snapshot, and logical time. | Service/integration |
| DTC recurrence | Increment occurrence count and version while preserving first-seen time. | Service |
| DTC ordering | Return DTCs in deterministic code order. | Service/API |
| Status filtering | Return records whose status mask intersects the requested mask. | Service/integration |
| Safe pagination | Enforce limit 200 and offset one million. | OpenAPI/API |
| DTC detail | Return stable not-found behavior for an unknown code. | API |
| Clear all | Remove all ECU DTCs for group FFFFFF. | Service/integration |
| Unsupported group | Return NRC 0x31 without deleting DTCs. | Service/API |
| Atomic evidence | Commit state, command, DTC mutation, audit, and outbox together. | Integration |
| Evidence minimization | Exclude snapshot measurements from audit and outbox payloads. | Service |
| RBAC read denial | Return 403 without diagnostics:read. | API/integration |
| RBAC write denial | Return 403 without diagnostics:manage. | API/integration |
| OpenAPI contract | Publish typed commands, results, bounds, and paths. | Contract |
| Migration lifecycle | Upgrade and downgrade all diagnostic tables. | Integration |
| Regression | Preserve ECU, CAN, identity, audit, and platform behavior. | Full suite |
| DID definition | Accept valid scalar types and reject mismatched initial values. | Schema/service |
| DID catalogue bound | Reject a 129th DID and enforce page limit 128. | Service/OpenAPI |
| Multi-DID read | Read one to sixteen unique DIDs in request order. | Service/API |
| Read session denial | Return NRC 0x7F when any DID disallows the active session. | Service/API |
| DID write authorization | Require writable metadata, permitted session, and manage RBAC. | Service/API |
| DID version concurrency | Reject stale session or DID versions with NRC 0x22. | Service/integration |
| DID value constraints | Reject type, numeric range, and string-length violations with NRC 0x31. | Schema/service |
| DID replay | Return persisted 0x22/0x2E evidence without repeated mutation. | Service/integration |
| DID evidence minimization | Prove audit and outbox never contain DID values. | Service |
| Routine schema bounds | Reject identifiers, durations, parameters, and result templates outside fixed limits. | Schema/OpenAPI |
| Routine catalogue bound | Reject a 65th routine and enforce page limit 64. | Service/OpenAPI |
| Routine start | Move idle state to running with ECU logical timestamps and incremented version. | Service/integration |
| Routine session policy | Return NRC 0x7F when the active session is not allowed. | Service/API |
| Routine stop capability | Return NRC 0x31 when stop is unsupported; otherwise stop at logical time. | Service/API |
| Routine stale version | Return NRC 0x22 without mutation when the state version does not match. | Service/API |
| Routine completion | Complete exactly at the declared ECU logical timestamp without sleeping. | Service/integration |
| Routine result | Return the bounded protected result only through the authorized response. | Service/API |
| Routine replay | Return persisted 0x31 evidence without repeated execution or version change. | Service/integration |
| Routine evidence minimization | Prove audit and outbox contain neither parameters nor result values. | Service |
| Routine atomicity | Commit definition/state/control, command, audit, and outbox evidence together. | Integration |
| Security command shape | Require a 16-character masked key only for sendKey and reject extra fields. | Schema/OpenAPI |
| Security session policy | Reject Security Access in default session with NRC 0x7F. | Service/API |
| Seed generation | Return a deterministic 16-character seed and 30,000-ms logical expiry. | Service/integration |
| Seed replay | Return the same protected seed without incrementing challenge or version twice. | Service/integration |
| Valid key | Unlock level 1, clear challenge state, and increment session/security versions. | Service/integration |
| Missing/expired seed | Reject sendKey with NRC 0x24 without unlocking. | Service/API |
| Invalid key | Persist a failed attempt and return NRC 0x35. | Service/API |
| Attempt ceiling | Third invalid key returns NRC 0x36 and starts a 10,000-ms logical delay. | Service/integration |
| Lockout delay | Return NRC 0x37 before the deadline and permit a new seed exactly at expiry. | Service/integration |
| Negative replay | Repeat a persisted denial without another failed-attempt increment. | Service/integration |
| Secret minimization | Prove raw key, seed, and key digest never enter audit or outbox evidence. | Service/security |
| Safe state query | Expose versions, attempt count, lockout, and challenge presence without secret material. | API/OpenAPI |
| Security atomicity | Commit failed attempt, command, audit, and outbox before returning the negative response. | Integration |
| Reset command shape | Publish subfunctions 0x01, 0x02, and 0x03 plus three bounded expected versions. | Schema/OpenAPI |
| Reset session policy | Reject every reset in default session with NRC 0x7F. | Service/API |
| Reset security policy | Reject hard and key-off/on reset below level 1 with NRC 0x33. | Service/API |
| Cross-volume reset | Prove UDS reset invokes ECU memory/boot/fault/time rules instead of duplicating them. | Service/integration |
| Diagnostic restoration | Restore default session, security zero, and empty challenge/lockout state. | Service/integration |
| Reset concurrency | Reject stale ECU, session, or security versions with NRC 0x22. | Service/API |
| Reset replay | Return stored 0x11 evidence without another boot, time advance, or version increment. | Service/integration |
| Reset atomicity | Commit ECU mutation, both evidence streams, diagnostic state, audit, and outbox together. | Integration |
| Download negotiation | Require programming session, level 1, matching versions, 16-bit range, and <=65,536 bytes. | Schema/service/API |
| Block bounds | Reject empty, malformed, larger-than-256-byte, or overflowing blocks. | Schema/service |
| Block sequence | Accept only the expected 0-255 byte counter and wrap 255 to 0. | Service/integration |
| Transfer concurrency | Reject stale transfer or ECU versions with NRC 0x22. | Service/API |
| Incomplete exit | Reject Transfer Exit before the declared byte count with NRC 0x24. | Service/API |
| Digest verification | Reject a mismatched SHA-256 without changing the ECU firmware version. | Service/integration |
| Flash activation | Update firmware profile and ECU version only after full verification. | Service/integration |
| Flash replay | Replay 0x34/0x36/0x37 without appending a block or activating twice. | Service/integration |
| Payload minimization | Prove raw image/block bytes never enter commands, logs, audit, or outbox. | Service/security |
| Flash atomicity | Commit transfer, firmware activation, command, audit, and outbox together. | Integration |

# 12. Implemented Evidence

- src/atep/diagnostics/ contains models, schemas, service, and router.

- Migration 0031_diagnostics_foundation owns session, command, and DTC persistence.

- Migration 0032_diagnostic_data_identifiers owns typed DID persistence and versioning.

- Migration 0033_diagnostic_routines owns routine definitions and execution states.

- Migration 0034_diagnostic_security_access owns the ECU Security Access challenge state.

- Migration 0035_diagnostic_flash owns the ECU firmware-transfer state.

- Events are atep.diagnostics.session.changed.v1, atep.diagnostics.dtc.reported.v1, and atep.diagnostics.dtc.cleared.v1.

- Permissions are diagnostics:read and diagnostics:manage.

- tests/test_diagnostics.py verifies contracts, logical time, versioning, replay, conflicts, evidence minimization, and permissions.

- API contract tests verify routes and safe pagination in generated OpenAPI.

- DID events are atep.diagnostics.did.created.v1, atep.diagnostics.did.read.v1, and atep.diagnostics.did.written.v1.

- Routine events are atep.diagnostics.routine.created.v1 and atep.diagnostics.routine.controlled.v1.

- Security Access events use atep.diagnostics.security.accessed.v1 without seed/key material.

- UDS ECU Reset reuses atep.ecu.reset.completed.v1 and adds atep.diagnostics.ecu.reset.v1 in the same transaction.

- Flash events are atep.diagnostics.flash.download.requested.v1, atep.diagnostics.flash.block.transferred.v1, and atep.diagnostics.flash.completed.v1; none contains raw firmware bytes.

# 13. Risks and Technical Debt

- V-1 through V-6 expose semantic UDS operations but do not yet encode wire PDUs.

- Status-mask meaning is retained as a byte; named bit helpers should be added with richer DTC queries.

- DTC creation is a controlled simulator boundary; automatic fault-to-DTC mapping is future work.

- The current clear operation loads at most 200 DTCs, matching the per-ECU V-1 storage bound.

- Security level 1 is implemented; additional OEM levels and protected production algorithms remain future work.

- Exact replay stores DID values in protected command evidence; retention and field-level access controls should be revisited before production telemetry use.

- Routine result templates are deterministic fixtures; future routines may require pluggable ECU behavior and explicit failure results.

- V-4 key derivation is transparent and deterministic for simulation; production security requires OEM algorithms, protected key material, hardware-backed execution, and threat analysis.

- V-5 returns a completed semantic reset response; transport adapters must later model response timing and ECU communication loss around physical reset execution.

- V-6 is a bounded simulator pipeline, not a bootloader or production flasher; signing, anti-rollback, erase/program routines, power-loss recovery, and OEM secure-boot policy remain future hardening.

# 14. Roadmap

V-1 through V-6 are implemented. The recommended next development is V-7: OBD-II compatibility, the DoIP transport boundary, diagnostic campaigns, and end-to-end scenarios that connect Digital Vehicle, ECU, CAN, Diagnostics, test execution, evidence, and Android Automotive visibility.

# 15. Study Exercises

- Explain why an ECU fault and a DTC are related but not identical.

- Calculate positive response service IDs for 0x10, 0x14, and 0x19.

- Compare NRC 0x22 and NRC 0x31 in the implemented operations.

- Trace a session-control request through RBAC, locking, versioning, audit, and outbox.

- Explain why exact command replay must not increment the session twice.

- Design a test for changed command-ID reuse.

- Decode status mask 0x09 into the UDS status-bit concepts you would expose next.

- Explain why DTC snapshot values are excluded from audit events.

- Design an automatic mapping from a confirmed BMS fault candidate to DTC 0A80FF.

- Propose a DID catalogue for battery SOC, SOH, temperature, and pack voltage.

- Define which DIDs may be written only in extended or programming session.

- Describe where ISO-TP segmentation belongs relative to the diagnostic service.

- Compare CAN/ISO-TP and Ethernet/DoIP as transport adapters for the same UDS domain.

- Design a negative test that proves an unsupported DTC group deletes nothing.

- Explain why simulation time produces more reproducible diagnostic tests than host time.

- Calculate the positive response identities for 0x22 and 0x2E.

- Explain why a DID version changes without changing the diagnostic-session version.

- Design a string DID with a maximum length and its negative tests.

- Compare NRC 0x31 for an invalid value with NRC 0x7F for the active session.

- Trace an exact 0x2E retry and prove the DID version increments only once.

- Calculate the positive response identity for Routine Control 0x31.

- Trace startRoutine from RBAC through session authorization, version checks, state, audit, and outbox.

- Explain why routine completion uses ECU logical time instead of a background timer.

- Design tests for stopRoutine when stop is supported, unsupported, or the routine is not running.

- Prove that a repeated start command does not increment invocation count twice.

- Compare definition version and execution-state version for an ECU routine.

- Explain why parameters/results are protected while status/version evidence may be shared.

- Design a battery-balancing routine and its allowed diagnostic sessions.

- Calculate the positive response identity for Security Access 0x27.

- Trace requestSeed and identify where the seed may and may not appear.

- Derive the simulator key for a sample seed and explain why the algorithm is not production-safe.

- Design a test proving raw keys never enter command results, audit, outbox, or logs.

- Compare NRC 0x24, 0x35, 0x36, and 0x37 in the implemented flow.

- Prove an exact invalid-key retry does not increment failed attempts twice.

- Advance ECU logical time to one millisecond before and exactly at lockout expiry.

- Explain why Security Access state and diagnostic-session security level use separate versions.

- Calculate the positive response identity for ECU Reset 0x11.

- Compare hardReset, keyOffOnReset, and softReset authorization in the implemented policy.

- Trace one accepted reset across ECU version/time/boot mutation and diagnostic state restoration.

- Prove an exact reset replay cannot increment boot count or simulation time twice.

- Explain why Volume V orchestrates rather than duplicates the Volume III reset lifecycle.

- Design rollback evidence for a failure after ECU mutation but before diagnostic evidence commit.

- Calculate the positive response identities for 0x34, 0x36, and 0x37.

- Explain why Request Download requires both programming session and Security Access level 1.

- Design block-sequence tests for duplicate, skipped, stale-version, and 255-to-0 wrap cases.

- Prove that raw firmware bytes do not appear in command, audit, outbox, or log evidence.

- Compare block SHA-256 evidence with the full-image digest verified at Transfer Exit.

- Trace a successful flash from negotiation through ECU profile/version activation.

- Design negative tests for incomplete images, overflow, digest mismatch, and stale ECU version.

- Explain which production concerns remain outside this simulator pipeline.

# 16. V-6 Review Worksheet

Use this worksheet after studying or demonstrating the increment:

- Record the tested firmware size, block size, number of blocks, and final SHA-256.

- Capture one successful 0x34 -> 0x36 -> 0x37 trace and its command/event identities.

- Capture one wrong-sequence, one incomplete-image, and one digest-mismatch result with NRC evidence.

- Verify that no raw firmware hex appears in logs, diagnostic commands, audit, or outbox records.

- Explain whether the observed result is sufficient for a simulator baseline and which production bootloader controls are still absent.